## Wooden surface restoration guide

## Surface maintanence

Surfaces treated with wood oils depending on environment conditions may need retreating as often as once a year.

Step-by-step guide

- Brush off dirt, dust and lose material from the surface. Use a spatula for hard particles or resin.

- Clean and wash mouldy spots with mould removal according to instructions. Wash vertical surfaces systematically from the bottom upwards to avoid runs. Let the surface dry.

- Mask with film and masking tape the surfaces not treated, e.g., steel structure.

- Carefully stir the paint. Cover the surface by using a painting brush. To avoid overlapping, apply the oil in continuous strokes to cover the whole surface evenly. Wipe off the excess oil from non-absorbing surfaces.

- Clean the brush with white spirit or appropriate cleaner. Dispose soaked material with care, it can be self-inflammable.

To be considered

- The final surface colour depends on the species, hardness and original colour of the wood, and the number of times it has been treated.

- Mould Removal lightens wood surfaces considerably.

- Test a small area of similar wood before application.

- Surfaces treated with oil will finally cure and harden after a week in normal conditions. After treatment, they will become dust dry in approx. two days and maybe cleaned gently with a soft brush, damp cleaning cloth or mop.

- Check the state of the treated surfaces annually but repaint it not too often to avoid the formation of a film on the surface.